Gazebo Simulation of a Four-Wheeled Robot (ROS 2)

### 1. Introduction

- A 4-wheeled mobile robot is designed and simulated using ROS 2 and Gazebo to study its motion, control, and sensor response.

- The robot model is described using XACRO/URDF files, which define its physical structure, joints, and wheel configuration.

- The simulation environment allows testing of navigation, line-following, and obstacle-avoidance algorithms without physical hardware.

- It provides a safe, cost-effective platform to validate robot design and control logic before real-world deployment.

### 2. Methodology / Process Overview (Flowcharts)

Flowchart 1 — Setup Sequence:

- Install necessary packages (publisher packages, xacro, gazebo-ros-pkgs, ros-core, geometry, gazebo msgs & plugins, teleop-twist-keyboard)

- Call Gazebo

- Create workspace (ws_mobile) and package (mobile_robot) — using ros2 pkg create --build-type ament_cmake mobile_robot

- Create subdirectories for launch and model

Flowchart 2 — File Creation Sequence:

- Create the URDF (Xacro) and Gazebo files — robot.xacro (defines geometry), robot.gazebo (additional Xacro file)

- Create a launch file — gazebo_model.launch.py

- Add dependencies — joint_state_publisher, robot_state_publisher, gazebo_ros, xacro, ros_gz_bridge

- Adjust CMakeLists.txt

### 3. Prior Requirements / Software Installation

List of dependencies installed via apt:

- gedit, ros-humble-joint-state-publisher (+ GUI), ros-humble-xacro, ros-humble-gazebo-ros-pkgs, ros-humble-ros-core, ros-humble-geometry2, ros-humble-gazebo-msgs, ros-humble-gazebo-plugins, ros-humble-ros-ign-bridge, ros-humble-teleop-twist-keyboard, gazebo

- Verified with gazebo --version

### 4. Implementation Steps

- Source ROS 2: source /opt/ros/humble/setup.bash

- Create workspace: mkdir -p ~/ws_mobile/src → colcon build

- Create package: ros2 pkg create --build-type ament_cmake mobile_robot

- Create subfolders: launch and model inside the package → colcon build

- Create URDF/Xacro and Gazebo files: robot.xacro, robot.gazebo

- Create Python launch file: gazebo_model.launch.py

- Add dependencies in package.xml: joint_state_publisher, robot_state_publisher, gazebo_ros, xacro, ros_gz_bridge

- Edit CMakeLists.txt to install the launch and model directories (add install(DIRECTORY launch model DESTINATION share/${PROJECT_NAME}) above if(BUILD_TESTING))

- Build and launch: colcon build → source ~/ws_mobile/install/setup.bash → ros2 launch mobile_robot gazebo_model.launch.py (opens the Gazebo model)

- Keyboard control: ros2 run teleop_twist_keyboard teleop_twist_keyboard

### 5. URDF & Xacro (Theory)

- The model folder contains the Xacro and URDF source files, along with a Gazebo empty-world model.

- URDF (Unified Robotics Description Format): a file format used to specify robot geometry and related properties.

- Xacro: an XML macro language used to parametrize and simplify URDF model development, reducing redundancy in robot description files.

### 6. Conclusion (suggested, to be written by you)

Summary of how the simulation validates robot motion/control logic in Gazebo before real-world deployment, and how URDF/Xacro enable modular robot description.

### 7. Flowchart and Screenshot